Technical Specifications for Development of Full Automated Project of Zhinvali HPP Process Operation

List of Zhinvali HPP Core Process Equipment:

- Hydroelectric installation (turbine РО-170-В-180, generator СВ 425/135-14), capacity  32,5 MW, 4 -sets;

- Generator circuit-breaker МГУ-20, 4 kV;

- Two sections of 10,5 kV bus bars, with two МГУ-20 type circuit-breakers;

- Power transformer ТД- 80000/220/10,5, 2 kV;

- Power transformer ТРДЦН- 63000/110/10,5;

- Power transformer ТМТН- 6300/110/10,5 ;

- Dividing transformer ТМ- 1000/10,5/10;

- Auxiliary service transformer ТСЗС 1000/10,  2 kV;

- Auxiliary service transformer ТСЗС 630/10,  3 kV;

- 220 kV, GL 314 F3 type electric gas breaker, 2 kV;

- 220 kV, S2DAT-245 type airblast circuit breaker with electric drive, 5 kV;

- 110 kV, ММО-110 type oil circuit breaker, 2 kV;

- РНД-110/1000 type airblast circuit breaker with manual drive, 5 kV.

Power plant one-line diagram  is attached.

It is necessary to inspect the facility on-site and to develop the following tentative design documentation:

- Reconstruction of generator cross-field excitation system on semiconductor rectifiers, with automatic control;

- Modernization of temperature control of generating units;

- Replacement of 110 kV circuit breakers by electric drive breakers, 5 kV;

- Continuous control of vibration of generating units;

- Continuous measurement of water flow in the turbines;

- Control butterfly valves;

- Control of hydro turbines by selecting optimal operation, process protections and alarm system;

- Control of generating units individually as well as in parallel regime;

- Electric and process protections of the generators. Measurement of electric parameters;

- Power transformer (T-1, T-2, T-3 and T-4) protections and alarm system;

- 220 kV breaker control (A-1 and A-2);

- Protection of 220 kV transmission line and measurement of electric parameters;

- Control of 110 kV breaker, protection and measurement of transmission line;

- Protection of generator bus bars;

- Protection of dividing transformer;

- Protections of auxiliary service transformers (1, 2, 3, 4 and 5);

- Control of 10 kV bus section breakers taking into consideration of automatic synchronization;

- Control of 220 and 110 kV airblast circuit breakers, alarm system, 10 kV;

- Interlocking electromagnet of circuit breakers;

- 10 kV feeder protections (four connections);

- Fault alarm of surface and underground parts.