Term of Reference

Plant: RMG GOLD

Site: Electrolytic bath

Kazreti Settlement, Georgia

2025

Initial Data

- Purpose of Purchase:

Replacing a worn-out electrolytic bath that has exhausted its resource.

- Name:

Electrolytic bath. Electrolytic liquid is being supplied continuously from the precious metals desorption cycle of the ion exchange resin regeneration process. The electrolytic bath should be equipped with a convenient bottom that allows for easy washing of cathodic deposits, has a containment device to calm the flow of the flowing solution, and also has a lid which is opened  vertically at 90°.

- Technical conditions:

| Name of materials | Internal/external/partial | Internal |
| --- | --- | --- |
| Minimal temperature | °С | 50 |
| Maximal temperature | °С | 98 |
| Flow rate in the electrolyzer | m3/h | 14 - 33 |
| Cycle |  | Locked |
| Electrolyzer type |  | Flowing out |
| Body material |  | Polypropylene |
| Description of liquid materials |  | Precious metals desorption cycle saturated solution, 30-40 g/l of NaOH, 15-25 g/l Using thiourea also contains 2.5-3% NaCN, pH-11.5-12.5 |
| Number of cathodes | Unit | 9 |
| Cathode material |  | SS-316 |
| Cathode frame area | m2 | 0.7 |
| Number of anodes | Unit | 10 |
| Anode material |  | SS-316 |
| Stripes material |  | Copper |
| Entrance gate | ∅ მმ | 76 |
| Exit gate | ∅ მმ | 76 |
| Solution drainage pipe | ∅ მმ | 60 |
| Pipe for cathodic residue discharge | ∅ მმ | 60 |
| Bath dimensions | length×width×height | 1600×850×860 |
| Height of bath legs | mm | 400 |

- Design requirements:

| Electrolysis bath | Requested |
| --- | --- |
| Feed, discharge and cathodic sediment discharge pipeline shut-off fittings | Requested |
| Drainage coating of gases and vapors produced during the electrolysis process | Requested |
| Cathodes and anodes | Requested |
| Bath operating instructions | Requested |
| Guarantee of working capacity | Requested |
| Important/critical spare parts | Requested |
| Spare parts stock for one year of operation | Requested |
| Special tool for installation | Requested |