Terms of Reference

of

Loading and unloading vehicle for the Bekthari deposit

Table of Content

Section  1. Common data

Sub-section  1.1 Name.

Sub-section  1.2. Geometric characteristics of the product.

Sub-section  1.3. References about the news.

Section  2. Usage scope.

Section  3. Operation terms.

Section  4. Technical requirements.

Sub-section  4.1. Key parameters and measures.

Sub-section  4.2. Main technical-economic and operational indicators.

Sub-section  4.3. Reliability requirements.

Sub-section  4.4. Requirements for power supply.

Sub-section  4.5. Requirement about the documentation.

Section  5. Requirements regarding the volume and/or submission deadline of warranties.

Section  6. Requirements for suitability for repair.

Section  7. Ecological requirements.

Section  8. Safety requirements.

Section 9. List of annexes

1. Common data.

1.1. Name

Loading and unloading vehicle.

1.2 Geometric characteristics of the product.

Height of the product: 5 000 mm. Width of the product: 5 000 mm. Minimal outer turning radius, mm: 8600 mm. Minimal internal turning radius, mm: 4400 mm. Maximum slope of the inclined ramp: 14°.

1.3 References about the news.

The supplier must guarantee that the delivered equipment is new, has not been used in the past, is a serial model, includes all the latest modifications, and has not been removed from production by the manufacturer at the time of delivery.

2. Usage scope.

A self-propelled forklift is used to move loads by lifting, transporting, and stacking them on another vehicle for further transport.

3. Operation terms.

Air temperature - from 15°C to +30°C.. Deposit inundation (humidity) 50 %. Air supply - 6000 მ³. Aggressive impurities in water рН 5-7. Air pollution - no. Pollination - 1.6.

4. Technical requirements.

4.1 Key parameters and measures.

Operation system Bucket capacity: 4 m3. Nominal load: 10,000 kg Empty weight: 29,000 kg Breaking strength: 188 KN Traction force: 200 KN Engine Model: TAD853VE Power: 235 kW @ 2200 rpm. Torque moment: 1064 NM @ at 1500 rpm. Types: Electro-reactive, water-cooled, turbocharged. Emission standard: Tier3Stage Ш Drive system Hydrotransformer: DANAC5000 series Gearbox (КПП): DANAR36000 series; Axle: Kessler D102 Break: Posi-Stop On-board Dimensions of the stripes:18.00-25 Cabin FOPS/ROPS certification Fully enclosed cabin with air conditioning and heating. Door with safety interlock. Two-lever control operation. LED lights front and rear. Dimensions Length: 9777 mm. Width: 2602 mm. Height: 2385 mm. Clearance: 380 mm. Unload height: 1914 mm. Turning angle: ±42.5°. Inner radius: 3211 mm. Outer radius: 6502 mm. Hydraulic system Hydraulic pump: Rexroth. Feedback according to load, double. Variable displacement pump, axial piston. Hydraulic valve: Rexroth, Parker, Mico Hydraulic tank capacity Hydraulic system: 240 l. Brake reverse chain. Fuel tank capacity: 310 l. Control system FET Electronic technologist. CAN-bus Control trunk. 5.7 Inch color display. Design module Remote control device within visible range. Malfunction alarm. Malfunction self-diagnosis. Record data loading. Others Centralized, manually operated lubrication system. Fire extinguisher. Arrow lock pin, joint axle, rear alarms. Reverse signal. Flashing beacon. Options Automatic centralized lubrication system. Centralized fire extinguishing system. Bucket shaking system. Bucket removal, removable teeth.

4.2. Main technical-economic and operational indicators.

Operation mode as required.

4.3. Reliability requirements.

Standard operating period – Not less than 4 years

4.4. Requirements for power supply.

Electrical equipment must be made with dust protection in mind. On-board network voltage – 24 V. Electric starter and generator – 24 V.

4.5 Requirement about the documentation.

- Operation, maintenance and repair instructions in electronic form. - Operation, maintenance and repair instructions in electronic and printed form. - Vehicle passport.

5. Requirements regarding the volume and/or submission deadline of warranties

Warranty service period – at least 12 months. Without mileage restrictions.

6. Requirements for suitability for repair.

Vehicle components and connections must withstand the forces and stresses that are exerted on them during active operation. The durability of the materials used must be appropriate to the intended use, taking into account the risks associated with fatigue, aging, corrosion and wear. The vehicle's operating manual (instructions) must specify the type and frequency of inspections and maintenance required to ensure safety. Wearable parts, components, and assemblies, criteria for their evaluation and replacement, must be specified.

7. Ecological requirements.

The exhaust gas content must comply with the Euro-5 standard.

8. Safety requirements.

The loading and unloading vehicle must be supplied in strict compliance with the "Safety Rules for the Transportation of Loose Materials".

9. List of annexes

N/A

Executed:

Senior mechanic	B. Statsenko   ______________