ტექნიკური დავალება

მისამართი:

მდინარე ხრამის მარჯვენა სანაპირო,ქვემო ქართლი,მარნეულის მუნიცეპალიტეტი, ნაკვეთი ს/კოდი 83.07.11.512;83.07.11.515  სოფელი დიდი მუღანლო.

დოკუმენტის ტიპი:

ქ. რუსთავის წყალმომარაგების, ხრამის სათავის პირველი აწევის მე-2-ე , მე-3-ე და მე-4-ე შახტურ ჭებში დაზიანებული ტუმბოების დემონტაჟი.

ტუმბოს ტექნიკური მონაცემები:

Q= 1200 მ3/სთ  H= 45 მ

ტუმბოს გეომეტრიული მონაცემები:

ტუმბოს სრული სიგრძე L= 18 მ  d= 400 მმ

სექციური შემადგენლობით  6X2,20 მ

2X1,20 მ

ტექ. დავალებით შესასრულებელი სამუშაოების კონცეფცია:

1.დაზიანებული ტუმბოს დემონტაჟი.

2.შახტურ ჭაში შემომავალი 9 სხივის დაბოლოვებაზე დამონტაჟებული d=200 მმ-იანი ურდულების სამართავი მილებისა(მყვინთავების პოზიცია) და საყრდენების დემონტაჟი.

3.ტუმბო სიღრმეში დამაგრებულია ოთხ ბაქანზე, შესაბამისად ბექნის და შესაბამისი დამჭერი რკინის კონტრუქციების  დემონტაჟი.

4.პირველ ბაქანზე დამცავი მუაჯირის მიწყობა L= 16 მ  H= 1 მ.

მითითება;

მითითებულ შახტურ ჭებში მუშაობისას უნდა იქნას დაცული შრომის უსაქრთხოების პირობები, საქართველოს კანონმდებლობით დადგენილი წესებისა და ნორმების მიხედვით.

შეადგინა:

ა. მაღრაძე  მობ: 599 659 114

04. 04. 2025 წელი.

Technical assignment

Address:

Right bank of the Khrami River, Kvemo Kartli, Marneuli Municipality, plot code 83.07.11.512; 83.07.11.515, village Didi Mughanlo.

Document type:

Dismantling of damaged pumps in the 2nd, 3rd, and 4th shaft wells of the first lift of the Khrami head of the city of Rustavi.

Pump technical data:

Q= 1200 m3/h H= 45 m

Pump geometric data:

Full length of the pump L= 18 m d= 400 mm

Sectional composition 6X2.20 m

2X1.20 m

Technical concept of the works to be performed under the task:

1. Dismantling the damaged pump.

2. Dismantling the supporting pipes and supports of the d=200 mm pipes installed at the ends of the 9 beams entering the shaft well.

3. The pump is fixed at depth on four platforms; its dismantling is accordingly.

4. Installation of a protective barrier on the first platform L= 16 m H= 1 m.

Note;

Labor safety conditions must be observed when working in the specified shaft wells, according to the rules and norms established by Georgian legislation.

Contact Person: A. Maghradze Mob: 599 659 114

04. 02. 2025.