Technical assignment of the drilling rig to be purchased

| 1 | Driver's cab | Only for moving of the rig |
| --- | --- | --- |
| 2 | Drilling mast | With adjustment of the height and inclination |
| 3 | Drilling diameter | 137 mm |
| 4 | Drilling | with pneumatic hammer |
| 5 | Diameter of the drilling pole | 102 mm or 114 mm |
| 6 | Length of the drilling pole | 6 m |
| 7 | Whole length of drilling | 200-250 m |
| 8 | Supply of the drilling pole | Hydraulically operated (robot) and desired to take a pole with the hoist |
| 9 | Storage of the drilling poles | On the platform of the drilling machine |
| 10 | (Drilling) supply system | 1-hydraulic cylinder and chain |
| 13 | Air system wiring | 80 BAR |
| 14 | Diesel engine | 350-400KW (desired to be CAT or Cummins) engine Required to be with cooling system with water |
| 15 | Lift and lower adjustment of the drilling machine | 4-with hydraulic cylinder (Out riggers) |
| 16 | Hoist | Cable Ф-10-11 mm L-30-50m. |
| 17 | Movement system | Tracked (with hydraulic engines) |
| 18 | Dust collection system | With automatic cleaning |
| 19 | Taking a sample | With sample separator hopper (cyclone) |