| # TECHNICAL SPECIFICATIONS REQUIRED |  |  | TECHNICAL SPECIFICATIONS OFFERED |  |
| --- | --- | --- | --- | --- |
|  |  |  | PLEASE FILL OUT ALL THE SPACES BELOW TO SPECIFY THE EQUIPMENT BEING OFFERED FOR THE OFFER TO BE VALID |  |
| Item |  | Description and purpose of the item | Make/ brand and model of item |  |
| Two wheel tractor |  | A walk behind 12 hp single axle equipped with a rotavator. |  |  |
| Unit |  | Tractor + rotavator + spares |  |  |
| Total quantity of units required |  | 16 | Name and address of the manufacturer of item |  |
| Country |  |  |  |  |
| Georgia |  |  |  |  |
| 1.0. | Technical Specifications: General |  |  |  |
| 1.1 | Heavy Duty Frame with accessories to enable attachment of mentioned implements. |  |  |  |
| 1.2 | Engine Power (rated – Continuous work): Min. 12 hp / 8.9 kW |  |  |  |
| 1.3 | Engine Type: Single Cylinder Diesel |  |  |  |
| 1.4 | Engine Starting Device: Hand cranking |  |  |  |
| 1.5 | Air Cleaner: Heavy duty type |  |  |  |
| 1.6 | Cooling system: Radiator or similar |  |  |  |
| 1.7 | Clutch: Single Dry Plate |  |  |  |
| 1.8 | Gearbox: Min 5 Forward and 2 Reverse Gears |  |  |  |
| 1.9 | Reverse gear fitted with safety device to prevent gear engagement when rotavator is working |  |  |  |
| 1.10 | Transmission: Gear drive in oil bath. |  |  |  |
| 1.11 | Fitted with differential lock or similar |  |  |  |
| 1.12 | Steering brakes: Differential/lockable type |  |  |  |
| 1.13 | Brakes: Drum type |  |  |  |
| 1.14 | Handle Bar Steering: Fitted with full controls and height/swivel adjustment, with shock absorbing mount. |  |  |  |
| 1.15 | Pneumatic drive wheels: Fitted with 6.50/80 x12 (4PR) tyres or similar. |  |  |  |
| 1.16 | Drive wheels to have adjustable track. |  |  |  |
| 1.17 | Ground Clearance: min. 210 mm |  |  |  |
| 1.18 | Working Weight, including trailer: Approx. 450 kg |  |  |  |
| 1.19 | Main Clutch Lever |  |  |  |
| 1.20 | PTO clutch lever |  |  |  |
| 1.21 | Throttle Lever |  |  |  |
| 1.22 | Steering/Brake Clutch Lever |  |  |  |
| 2.0 | Technical Specifications: Engine |  |  |  |
| 2.1 | Rated power: 12 hp (8.9 kW) |  |  |  |
| 2.2 | Fuel type: Diesel |  |  |  |
| 2.3 | Displacement: 1000 cm3 app. |  |  |  |
| 2.4 | Rate speed: 2000 rpm app. |  |  |  |
| 2.5 | Specific fuel consumption: 300 g/kWh |  |  |  |
| 2.6 | Fuel consumption when ploughing: max. 1.3 l/h |  |  |  |
| 3.0 | Technical Specifications: Rotavator |  |  |  |
| 3.1 | Working Width: 600 – 720 mm |  |  |  |
| 3.2 | Working Depth: Up to 150 mm |  |  |  |
| 3.3 | Drive wheel height adjustable to the required depth of cut |  |  |  |
| 3.4 | Transmission: Chain and sprocket and/or gears |  |  |  |
| 3.5 | Blades: Blended |  |  |  |
| 3.6 | Number of Blades: 18 or more |  |  |  |
| 3.7 | Rotor Speed: 180 - 200 rpm |  |  |  |
| 3.8 | Fitted with a riding chair |  |  |  |
| 4.0 | Manuals: |  |  |  |
| 4.1 | One operator’s handbook and manual in English must be supplied for each set (tractor and implement), preferably both in hardcopy. |  |  |  |
| 4.2 | The cost of the manuals must be included in the DAP/DAT cost of the equipment. |  |  |  |
| 5.0 | Spare parts: |  |  |  |
| 5.1 | A list of Manufacturer’s recommended, fast-moving service and wearing spare parts for 2 years of operation must be submitted for each tractor. The list must be individually itemised, costed and submitted with your bid. The list must include at least the following: |  |  |  |
| 5.1.1 | Standard pulley (size would depend on the engine horsepower and speed) |  |  |  |
| 5.1.2 | Belt pulley cover |  |  |  |
| 5.1.3 | V-belt accessories |  |  |  |
| 5.1.4 | Spark plug |  |  |  |
| 5.1.5 | Fuel filters |  |  |  |
| 5.1.6 | Oil filters |  |  |  |
| 5.1.7 | Oil Seals |  |  |  |
| 6.0 | Accessories |  |  |  |
| 6.1 | Tool box with standard kit (tools) for minimum repair requirements including the following: |  |  |  |
| 6.1.1 | Open wrenches corresponding to engine bolt size |  |  |  |
| 6.1.2 | Allen wrench set |  |  |  |
| 6.1.3 | Adjustable wrench |  |  |  |
| 6.1.4 | Positive and negative screwdrivers |  |  |  |
| 6.1.5 | Pliers |  |  |  |
| 6.1.6 | One set filler gauges |  |  |  |
| 7.0 | Compliance of norms or standards |  |  |  |
| 7.1 | All rotating and fast moving parts must be fully guarded |  |  |  |
| 8.0 | ##### Assembly, inspection and after sales service: |  |  |  |
| 8.1 | -Option 1: Through a local authorized distributor/agent (Preferred): Kindly indicate the name, Tel./Fax number and email or postal address of your service agents in the destination country or neighbouring ones. |  |  |  |
| 8.2 | Before shipment or delivery the supplier must undertake assembly and final inspection. |  |  |  |
| 8.3 | Tractors must be delivered fully assembled and in order to be ready to use. |  |  |  |
| 8.4 | The bid price must include the cost of delivery |  |  |  |
| 9.0 | Warranty: |  |  |  |
| 9.1 | Preferred option: A full and clear manufacturer’s warranty statement transferable with the ownership of the equipment must be submitted with your bid detailing the terms and conditions that are being offered. The minimum requirement is for a twelve months warranty period or the first 2,000 hours of operation. The services required under this coverage will be provided by the local authorized distributor/agent |  |  |  |