Rehabilitation of washroom in public school of village Zumi, Chkhorotsku Municipality Arrangement of a new well and reservoir

Explanatory Note

The project includes arrangement of a well and reservoir on the territory of the public school of village Zumi, Chkhorotsku Municipality, also, the rehabilitation of the existing washroom. The place is selected to arrange the well and tank. The draft of the drawing is made.

In order to improve water supply, it is necessary to dig out a well, arrange a stand with metal square tubes (according to the drawing) and install 2,5 t. tank on it; install a new water pump in the well and connect to the tank with pipeline; arrange power supply; the pipes and tanks will be wrapped with thermal insulation material. The existing washroom roof will be dismounted and a new one will be installed with the material agreed with the customer. High quality metal-plastic doors and windows will be installed; the windows shall open and hang out. The existing sinks, squat toilets and concrete urinals will be dismounted; damaged daub of walls will be removed and the walls will be covered with sand-cement solution. The inner walls will be treated with spackle and will be painted with silicone antiseptic paint; sinks, squat toilets and special toilets for pupils with disabilities – one in the male washroom and the other in the female washroom. The building facade will be splashed and painted in the colour agreed with the customer. Drainage will be arranged around the washroom building; the stairs will be tiled with mosaics and a special ramp with simple construction will be arranged on the stair for pupils with disabilities (ridge of 10cm width will be fixed on it).

The executor shall take into consideration all extra charges (income, pension, overhead, profit, unforeseen expenses, transportation, machine mechanisms) defined by Georgian legislation, in the unit price of the works to be executed in the estimate.

1. Environmental protection measures

In the process of work, attention shall be paid to the proper operation of the oil-lubrication system of vehicles and mechanisms to minimize the risk of environmental pollution, and in case of such a fact, the contaminated place shall be immediately localized, useless ground shall be removed and taken to the specified site.

2. Construction organization project

The number of mechanisms and workers required to perform individual works shall be determined according to the relevant construction norms and rules.

All types of construction - installation and transportation works shall be carried out in compliance with the requirements of the safety equipment.

1. Before the start of ground works, it is necessary to define and separate the working area and find out whether any kind of underground communications pass through separate areas;

2. The executing personnel shall be informed about ground processing methods and technology;

3. The construction site shall be immediately fenced and marked with appropriate warning signs and with reflective signs at night;

4. The personnel working on concrete works shall be provided with special instructions on concrete works and be informed about the technological properties of concrete.

5. The norms and rules of the labour security shall be followed by the personnel working on all kinds of work.

6. When working with the products, the following shall not be allowed:

- Personnel without appropriate experience and qualification;

- Drunk personnel;

- Personnel shall be informed about the toxicity and other technical properties of the materials;

7. The personnel shall be equipped with special work uniform, helmets, glasses and other necessary equipment.

8. The site shall be provided with first aid medicines and dressings.

In case of injury to the personnel working on the site, he/she shall be provided with on-site first aid and in case of serious injury, he/she shall be taken to the relevant medical institution.