Arrangement of sewage system for paint factory of

Village Ingiri, Zugdidi Municipality

Explanatory Note

The project includes arrangement of a new sewage system in village Ingiri, Zugdidi Municipality. The area was selected and the location for placing the pipeline system on this territory was selected. The geodesic signs were taken, draft of drawings was made.

To arrange the sewage system, it is necessary to carry out the following construction works: excavation of a trench; arrangement of sand bed; installation of plastic pipes with appropriate inclination, filled with ballast; arrangement of the wells according to the drawing.

2. Construction organization project

The purchased materials shall be accompanied with the certificates of compliance.

The number of mechanisms and workers required to perform individual works shall be determined according to the relevant construction norms and rules.

All types of construction - installation and transportation works shall be carried out in compliance with the requirements of the safety equipment.

1. Before the start of ground works, it is necessary to define and separate the working area and find out whether any kind of underground communications pass through separate areas;

2. The executing personnel shall be informed about ground processing methods and technology;

3. The construction site shall be immediately fenced and marked with appropriate warning signs and with reflective signs at night;

4. The personnel working on concrete works shall be provided with special instructions on concrete works and be informed about the technological properties of concrete.

5. The norms and rules of the labour security shall be followed by the personnel working on all kinds of work.

6. When working with the products, the following shall not be allowed:

- Personnel without appropriate experience and qualification;

- Drunk personnel;

- Personnel shall be informed about the toxicity and other technical properties of the materials;

7. The personnel shall be equipped with special work uniform, helmets, glasses, etc.

8. The site shall be provided with first aid medicines and dressings.

In case of injury to the personnel working on the site, he/she shall be provided with on-site first aid and in case of serious injury, he/she shall be taken to the relevant medical institution.